# REVISION

| Issue | Comentarios | Fecha edición |
| --- | --- | --- |
| - | Creation. | 28/05/2013 |
| A | Reduced Envelope (for Depot) | 25/06/2013 |

# OBJECT

The present document describes the maximum developed kinematic envelope in curves of the unit proposed by CAF in different curve radius.

The developed kinematic envelope given in the present document takes into account the geometric curve overthrow and maximum dynamic displacements of the vehicle, and can be used to be compared to the infrastructure elements considering an additional safety clearance.

# MAXIMUM KINEAMTIC ENVELOPE IN CURVES

Maximum developed kinematic envelope in the height of the vehicle is given in table 1 for different curve radius in the inner and outer side of the curve.

Figure 1 shows the straight to curve transition geometry to be cleared for the tramway.

# REDUCED KINEAMTIC ENVELOPE IN CURVES

Reduced developed kinematic envelope in the height of the vehicle is given in Table 2 for different curve radius in the inner and outer side of the curve.

This envelope is applicable to areas where the speed of circulation is low and under control, with no passenger service as inside de depot and maintenance facilities.

Ei

Ea

14 m

2 m

8 m

Figure 1	Straight-Curve transition geometry

|  | 19 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 100 | 150 | 200 | 300 | 500 | 600 | 800 | 1000 | 2000 | STR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ea (*) | 1986 | 1962 | 1871 | 1809 | 1732 | 1685 | 1653 | 1614 | 1591 | 1559 | 1544 | 1528 | 1515 | 1512 | 1508 | 1506 | 1501 | 1440 |
| Ea (**) | 1880 | 1856 | 1764 | 1702 | 1625 | 1578 | 1547 | 1507 | 1484 | 1452 | 1437 | 1421 | 1408 | 1405 | 1401 | 1399 | 1394 | 1340 |
| Ei | 1672 | 1653 | 1580 | 1533 | 1473 | 1437 | 1414 | 1384 | 1366 | 1351 | 1345 | 1340 | 1340 | 1340 | 1340 | 1340 | 1340 | 1340 |

Table 1	Developed Kinematic Envelope

|  | 19 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 100 | 150 | 200 | 300 | 500 | 600 | 800 | 1000 | 2000 | STR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ea (*) | 1843 | 1819 | 1728 | 1666 | 1589 | 1542 | 1510 | 1471 | 1448 | 1416 | 1400 | 1385 | 1372 | 1369 | 1365 | 1363 | 1358 | 1354 |
| Ea (**) | 1736 | 1712 | 1621 | 1559 | 1482 | 1435 | 1403 | 1364 | 1341 | 1309 | 1293 | 1278 | 1265 | 1262 | 1258 | 1256 | 1251 | 1247 |
| Ei | 1609 | 1590 | 1517 | 1469 | 1410 | 1374 | 1351 | 1321 | 1303 | 1280 | 1268 | 1256 | 1247 | 1247 | 1247 | 1239 | 1247 | 1247 |

Table 2	Reduced Developed Kinematic Envelope

(*) Rearview camera included

(**) Rearview camera not included